CR-BIS: Continuous Recording Beam Image Shift Acquisition for Cryo-EM

Version: 2.0
Platform: SerialEM + Python (≥3.4)

License and Usage Terms

This software is **for non-commercial academic research use only**. Commercial use is strictly prohibited as the underlying method is the subject of a pending patent. A detailed custom license is provided in the `LICENSE` file.

Overview

CR-BIS (Continuous Recording Beam Image Shift) is a data acquisition strategy for cryo-EM and cryo-ET that minimizes camera-induced delay and maximizes throughput.
It extends the standard Beam Image Shift (BIS) method by continuously recording multiple target points in a single exposure, enabling up to 2–4× faster single-particle acquisition and ~3× faster tomographic collection without compromising image quality.

This repository provides:

- SerialEM scripts for CR-BIS data acquisition

- Python preprocessing tools for frame splitting and metadata generation

- Example test datasets and reference parameters

Requirements

Hardware

- Transmission Electron Microscope (e.g., Titan Krios)

- Direct Electron Detector (tested: Gatan K3, Falcon 4i, Falcon4)

- SerialEM control computer (Windows)

Software

| Component | Version / Notes |
| --- | --- |
| SerialEM | ≥ 4.1 |
| Python | ≥ 3.4 |
| MotionCor2 | For frame alignment |
| Warp / RELION / AreTomo1 | Optional downstream processing |

Repository Structure

CR-BIS/

│

├── CR-BIS _scripts_v2/

│   ├── Report_Clock_script.txt				# Measure camera delay curve

│   ├──np_81kx_20260112_script.txt

│   		├── parameter                 # Core parameter configuration file

│   		├── take_map                  # Low-mag map acquisition

│   		├── Tomo_group_V_p1          # Pre-alignment script

│   		├── Tomo_group_X             # Main CR-BIS acquisition

│   		├── reset_X                    # Restore microscope settings

│   └── file_checker.py # Additional control for the advance movement of stage

│

├── data_process/

│   ├── Falcon/		# Basic command lines of preprocess for CR-BIS-Tomo

│   		├── calculate_sep_range_python34.py

│		│		# calculate blank frame gap before splitting EER files

│   		├── eer_writer_5_v4_py34.py

│		│		# eer splitting script

│   		├── tomo_log_edit.py

│		│	# updated information from *log_sep.txt file and modifies tomo_log.txt

│   		├── mdoc.py    # generate the .mdoc file for each tilt series

│		└── tomo_log.txt  #  tomo_log.txt example

│   └── K3/

│   		├── group_single_point_tif_MotionCorr2_parallel_v2_tiffile.py

│   		├── modi_tif_log,py

│   		├── modi_tomo_log.py

│		├── mdoc.py

│		└── tomo_log.txt  #  tomo_log.txt example

└── CR-BIS_User_Guide_v2.0.pdf		# Full illustrated manual

Basic Workflow

Part 1 — Pre-Calibration

- Use Report_Clock_script.txt to measure total record time vs. exposure setting time.

- Determine total exposure formula for your detector model (see guide).

Part 2 — Data Acquisition in SerialEM

- Configure parameters in parameter.txt.

- Acquire low-magnification maps (take_map.txt).

- Select target points in Navigator.

- Run sequentially:

  - Tomo_group_V_p1.txt (pre-alignment)

  - Tomo_group_X.txt (CR-BIS data acquisition)

Part 3 — Preprocessing

- For TIF data:

- python3 group_single_point_tif_MotionCorr2_parallel_v2_tiffile.py /path/to/raw_data

- For EER data: python3 eer_writer_5_v4_py34.py frame_num

- For Cryo-ET: generate .mdoc files based on tomo_log.txt using mdoc.py.